Music 451C Exercise Data Analysis

2026-01-31 Takako Fujioka

Overview

1. Curry 7 Splitting files to two individual files and Data conversion

2. Transfer and Copy files to CCRMA terminal or your computer

3. Brainstorm via MATLAB scripts for Preprocessing and Exporting

4. Further Analysis and Plot in MATLAB

5. Stats in R or MATLAB

- Step 1 & 2 → see ‘EEG files convert & transfer.pdf’ in Canvas

- EEG raw (.cnt) data are:

- https://ccrma.stanford.edu/~takako/451C/Piano_Duet2017/

Or

- use SFTP (use your FTP client such as Filezilla, or SCP command at a Mac OS command line to ccrma-gate.stanford.edu

- scp -r takako@ccrma-gate.stanford.edu:/user/takako/Library/Web/451C/Piano_Duet2017 (your local destimation folder name)

- /user/t/takako/Library/Web/451C/Piano_Duet2017

  - These are the data used for Huberth et al. 2019 and Washburn et al. 2019

  - S01-S24 are paired at the time of recording like S01_S02, S03_S04, etc. and Odd-numbered subject is ultimately called SubA, while even-numbered subject is SubB.

- This handout explains Step 3-4 in detail.

- If a MATLAB script has ‘bst’ in the filename, make sure Brainstorm with the protocol in question is open.

Step 3. Brainstorm via MATLAB scripts for Preprocessing and Exporting

In Brainstorm make 'Duet2017' protocol.

Copy the brainstorm preprocessing scripts from my directory

(/user/t/takako/MATLAB/Duet/eeg_analysis/451CW26_exercise) to your MATLAB working diretory (e.g., MyHome/Documents/MATLAB/451C_W26_Exercise) and run them in the following order.

1) do_0a_bst_linkraw_DuetEx.m

- Your cnt file location should be correctly reflected in the script.

  - Line 19 mydirname

  - mydirname = "C:\Users\tfujioka\Documents\MATLAB\451C_W26_practice\raw\Piano_Duet2017"

- Identify which pair, which melname are un-commented (be consistent between them)

  - Line 24-36 for pair

  - Line 44-56 for melname

- Then from MATLAB EDITOR tab, Run

- You will see for each subject 8 files will appear in Brainstorm as ‘Link to Raw’

- This will make a whole bunch of _.cnt files to be linked in Brainstorm database. Make sure your brainstorm is open with the Duet2017 protocol.

- However, this will create the ‘CNT 2D’ setup in the channel file automatically, which your topomaps in the brainstorm don’t have accurate information of the electrode locations.

- Therefore, after you run the script, for each subject, you have to import channel files from your fresh default 'channel_initial.mat' (make edit from Colin27-Neuroscan 68, and make a copy somewhere in your brainstorm_db and rename it as ‘channel_initial.mat’)

2) do_0b_bst_ssp_DuetEx.m

- The same principle should be applied for ‘pair’ and ‘melname’ in order to correctly identify filenames

- This script takes one of the raw files for each subject and runs SSP procedure. After running this, you should check all the projectors and modify the selection.

3) do_1a_add_trigger_DuetEx.m

- Similarly, you should care about ‘pair’ and ‘melname’ before running

- Makesure the folder ‘EventFiles’ are created under ‘451C_W26_practice’ or whatever your work directory so that the output of this program (8 event files per person) would be written

  - Line 686 event_filename

- This script reads trigger information from the brainstorm info about each raw file, and recognize which condition the block was, and give a set of new names for a certain group of trigger codes then writes out 8 event files per person outside of Brainstorm first

- For each file, 5 new markers in total will be created, for example, if this block is Same melody and Human partner,

◦ DevSelfSameHuman

◦ DevOtherSameHuman

◦ StdSelfSameHuman

◦ StdOtherSameHuman (these four are used for FRN analysis)

◦ LeaderSameComp (for the onset of Phrase 5 for alpha analysis)

- Trigger codes about combination of partner/score was presented at the beginning of each trial (first metronome count in) whereas any sound produced thereafter was associated with a number that tells us which position of the note is and whether that was standard or deviant.

- Success trial is marked as 81, Error trial is marked as 80 sent from Max Patch

- This script just runs and returns to the MATLAB command line. No output is made.

- Therefore, you must see whether the new files are correctly made under EventFiles folder.

3) do_1b_bst_import_trigger_DuetEx.m

- This script now imports the event information from these event files to Brainstorm.

- Care about ‘pair’ and ‘melname’ before running

- Make sure your eventfile folder is correctly identified before running

  - Line 84 event_filename

- When you open a raw file, you will see the five new markers in the right side of the Brainstorm event window

4) do_1c_bst_import_continuous_DuetEx.m

- This script now imports the raw files into Brainstorm (this is preparation necessary for the next do_1d_bst_run_repair_steps)

- Care about ‘pair’ and ‘melname’ before running

- Each ‘Link to raw’ file would be imported as is to ‘Raw’ file in Brainstorm

5) do_1d_bst_run_repair_steps_ DuetEx.m

- Make sure you have ‘repair_steps_12_data2.m’ in the same folder as these MATLAB　scripts, or in your MATLAB folder with Path setup

- Care about ‘pair’ and ‘melname’ before running

- This script now clean up the drift gaps that occur every 10 sec in .cnt file.

Now we have two types of analysis, FRN and AlphaERD.

FRN

6-1) do_2a_bst_epoch_avg_DuetEx.m

- This script runs epoching, peak-to-peak bad channel rejection, and averaging for Standards and Deviants.

- Care about ‘pair’ and ‘melname’ before running

- Identify where the brainsgtorm protocol is in your filesystem

  - Line 45 dirname

- This step takes most of the time, especially when you want to process data from multiple participants. You could leave logged in and let it run for a few hours – overnight.

7-1) do_3a_export_data_FRN_DuetEx.m

- This step reads all the workspace of averaged ERPs from the brainstorm_db and store into MATLAB variable for further analysis, and save in a workspace. You need to change the filename for your location and possibly today’s date.

  - Line 10 dirname (for Brainstorm protocol_name/data)

  - Line 12 subjname_all

  - Line 90 mydirectory (storing this user workspace holding the data exported from Brainstorm)

  - Line 92 mydate (it gives you the way to do the data version control)

- This step is very quick, and does not require Brainstorm open.

Alpha ERD

6-2) do_2b_bst_epoch_avg_ alphaERD_DuetEx.m

- Care about ‘pair’ and ‘melname’ before running

- Identify where the brainsgtorm protocol is in your filesystem

  - Line 59 dirname

- This script runs downsampling, epoching, peak-to-peak bad channel rejection, and averaging for Follower and Leader at the epochs around the measure 5 onset (-1.5 to 3.0).

- The resultant evoked responses are not used for the subsequent analysis explictly, but it is important to have a good quality data, because subsequent process still uses the evoked responses.

- Therefore, it is important that you inspect the evoked responses, and if that’s noisy, it is necessary to go back inspection of single trials, and do ‘bad channel rejection’ and/or ‘reject trial’ depending on the nature of the noise.

- If a sudden vertical jump happens on the most of channels, then you should reject that trial.

- If something happens on only part of the channels, then you should do ‘bad channel rejection.

- If you did such manual rejection, then you need to average that condition manually.

- It’s recommended that you keep notes about these manual navigations (subject, condition, trial, why you reject what)

7-2) do_3b_bst_ induced_alphaERD.m

- Care about subj

- Identify where the brainstorm protocol is in your filesystem

  - Line 47 dirname

- This script runs the time frequency analysis on these single trials and standardize to obtain ERD (%) value

- To get ERD/ERS value, you need to specify ‘baseline’ which would provide ‘100%’ for power comparison (e.g., expressing the power fluctuation at a given time point as how much of the power is observed compared to the ‘base-line’ window’s power value)

- Average power is averaged across single trial time frequency maps

7-3) do_3c_export_data_alphaERD_DuetEx.m

- This exports the alpha data to MATLAB workspace for subsequent analysis

- Care about subj

- Line 55, dirname

- Line 152, mydate for the filename of the workspace we are saving

Step 4. More processing

do_4a_analysis_FRN_DuetEx.m

- This script reads the workspace and do analysis steps

- Reorganize the datasets

- Band-pass filtering

  - prepare low-pass filtered version for visualization (plotting) while statistics on amplitude can be done with the filtered data

  - Usually, for ERPs (evoked responses) you could do somewhere between 25-40 Hz, depending on the epoch length and the spikiness of the peak of the target response.

  - For high-pass, usually you don’t (or shouldn’t) do that. If there is a lot of linear trend (e.g., straight line drift) happening in the data (especially when your epoch is long) you might need to do 0.1, 0.3 or 0.5 Hz.

- baseline-DC-offset at any time window of choice

- obtain difference waveform by subtraction (deviant – standard)

- Electrode grouping at any number of groups and choice of electrodes

  - specify the scalp areas of your interest where the target response is largest, and interpretable) then identify the indices of the electrode (see the Brainstorm electrode file)

  - do_make_elec_groups.m

  - Typically you try some different configuration

  - For the auditory evoked responses, fronto-central left, midline, right may be the usual way to start, and do some adjustment.

  - When you have multiple groups, then your statistics should include ‘electrode grouping’ as one within-subject factor for repeated measures ANOVA (analysis of variance)

- Preliminary t-test

  - simple t-test in every timepoint for contrasts-of-interests

  - when you have a particular contrast (e.g., Deviant vs. Standard) you can do t-test in a time-point wise manner. This gives a preliminary area in time for identifying target components, and the respective time windows to get amplitude/latency for statistics

- Plotting

  - plotting lines for ERP and difference waveforms

  - Usually we plot ‘grand average’ (all subjects in) but you need to select subjects so that you exclude extra noisy or very outlying people’s data.

  - Therefore, it’s important to plot individual data to check thoroughly the integrity of the data for each person, each condition, each electrode grouping.

- Amplitude calculation

  - when you have identified the time window and electrode group to examine statistics, you need to do the amplitude extraction. We normally use ‘mean area amplitude’ within a certain time window. Such time windows can be centered around the grand average peak latency.

  - determine peak latency and the time-window for individual mean amplitude calculations (50% of peak-to-peak amplitude before and after the major peak such as FRN and P3a) based on the grand average of difference waveforms averaged across all conditions

    - this calculation (Line 619-699) should be done line by line to make sure that the peaks or points are correctly identified. Not very automated.

  - Writing out a text file of individual mean amplitude values for the R statistics such as ANOVA or regression

    - Line 703 ‘mydate’

- Save the workspace (also using ‘mydate’ for the version control)

- Plot topomaps (make sure you have ‘rri_topoplot.m’)

(Alpha analysis to be described in the next handout 2026-01-31)